2 Phase stepper motor driver JK0220

Characteristics：

1.DC power input type:12V~36VDC ,

2.Output current:0.25A-2.0A

Mircostepping:1(1.8º)、1/2、1/4、1/8、1/16、1/32、1/64、1/128

Pulse/rev: 200,400,800,1600,3200,6400,12800,25600

3.Dimensions：96mm×60mm×24.5mm

4.Weight：<200g.

5.Working environment：Temperature-15～80℃

I/O Ports：

1.GND,：VDC+: it input DC power

Note:Must guard against exceeding 36V,

so as not to damage the module

2.A+、A-：Stepping motor one winding

3.B+、B-：Stepping motor other winding

4.PUL ：Stepping pulse input

5.DIR：Stepping motor direction input

6.VCC：power supply port for inside optical coupling

7.EN: motor free

NOTE：

1. When ambient temperature is high or working current over 1.05A,  fix the module on big metal shell , or use axle flows fan dispels the heat, to make the module run reliably for a long time.

2. Half current automatically: if control machine not send out signal  in half second, driver enter half current state of automatically for electricity saving, the phase current of the winding of the electric

Switch Choice:

1、Microstepping choice:

| SW5 | On | Off | On | Off | On | Off | On | Off |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SW6 | On | On | Off | Off | On | On | Off | Off |
| SW7 | On | On | On | On | Off | off | Off | Off |
| Micro | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 |

2、Current choice：

| SW1 | Off | On | Off | On | Off | On | Off | On |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SW2 | Off | Off | On | On | Off | Off | On | On |
| SW3 | Off | Off | Off | Off | On | On | On | On |
| Current（A） | 0.25 | 0.5 | 0.75 | 1.00 | 1.25 | 1.50 | 1.75 | 2.00 |

3、installation dimension